December 2024

19 W. Main St

Hartford, MI   49057

Phone: 269-621-2477

Fax:  269-621-2054

cityclerk@cityofhartfordmi.org

www.cityofhartfordmi.org

###### CITY OF HARTFORD

To:  Water Customer’s

Re:  Freezing Water-lines Policy

The Department of Public Works will respond to assist you in thawing out your pipes one time at no charge; the second response will be at a cost of $50.00 plus time and materials.  If your meter freezes and breaks and cannot be repaired there will be a charge to replace it.

It is the responsibility of the home owner to protect their water lines from freezing. We suggest you run your water at a flow that is about the size of a pencil.  If you run your water to keep your lines from freezing, document when you turn the water on and shut the water off with an explanation and turn into City Hall for possible credit if it is above your normal/average usage.

DO NOT ATTEMPT TO THAW CITY OF HARTFORD WATER/SEWER LINES; CALL THE DEPARTMENT OF PUBLIC WORKS EMERGENCY NUMBER AT 269- 208-5238

- Why Pipe Freezing is a Problem:  Water has a unique property in that it expands as it freezes. This expansion puts tremendous pressure on whatever is containing it, including metal or plastic pipes. No matter the "strength" of a container, expanding water can cause pipes to break. Pipes that freeze most frequently are those that are exposed to severe cold, like outdoor hose bibs, swimming pool supply lines, water sprinkler lines, and water supply pipes in unheated interior areas like basements and crawl spaces, attics, garages, or kitchen cabinets. Pipes that run against exterior walls that have little or no insulation are also subject to freezing.

- Preventing Frozen Pipes :  Before the onset of cold weather, prevent freezing of these water supply lines and pipes by following these recommendations:

- Drain water from swimming pool and water sprinkler supply lines following manufacturer's or installer's directions. Do not put antifreeze in these lines unless directed. Antifreeze is environmentally harmful, and is dangerous to humans, pets, wildlife, and landscaping.

- Remove, drain, and store hoses used outdoors. Close inside valves supplying outdoor hose bibs. Open the outside hose bibs to allow water to drain. Keep the outside valve open so that any water remaining in the pipe can expand without causing the pipe to break.

- Check around the home for other areas where water supply lines are located in unheated areas. Look in the basement, crawl space, attic, garage, and under kitchen and bathroom cabinets. Both hot and cold water pipes in these areas should be insulated.

- Consider installing specific products made to insulate water pipes like a "pipe sleeve" or installing UL-listed "heat tape," "heat cable," or similar materials on exposed water pipes. Newspaper can provide some degree of insulation and protection to exposed pipes – even ¼” of newspaper can provide significant protection in areas that usually do not have frequent or prolonged temperatures below freezing.

# To Thaw Frozen Pipes:

- If you turn on a faucet and only a trickle comes out, suspect a frozen pipe. Likely places for frozen pipes include against exterior walls or where your water service enters your home through the foundation.

- Keep the faucet open. As you treat the frozen pipe and the frozen area begins to melt, water will begin to flow through the frozen area. Running water through the pipe will help melt ice in the pipe.

- Apply heat to the section of pipe using an electric heating pad wrapped around the pipe, an electric hair dryer, a portable space heater (kept away from flammable materials), or by wrapping pipes with towels soaked in hot water. Do not use a blowtorch, kerosene or propane heater, charcoal stove, or other open flame device.

- Apply heat until full water pressure is restored. If you are unable to locate the frozen area, if the frozen area is not accessible, or if you cannot thaw the pipe, call a licensed plumber.

- Check all other faucets in your home to find out if you have additional frozen pipes. If one pipe freezes, others may freeze, too.

# Future Protection :

- Consider relocating exposed pipes to provide increased protection from freezing.

- Pipes can be relocated by a professional if the home is remodeled.

- Add insulation to attics, basements and crawl spaces. Insulation will maintain higher temperatures in these areas.

- For more information, please contact a licensed plumber or building professional.

Please let us know if you have any questions or concerns.